# Reference TX Chain

# Purpose

This document lists the components and characteristics of the reference RF chain used by the Agentic AI Antenna-to-bits demo. It is intended to be read by the demo agent before building the MATLAB RF budget and waveform-analysis workflow.

# Global Parameters

| Parameter | Value |
| --- | --- |
| IF frequency | 4 GHz |
| LO frequency | 8 GHz |
| RF frequency | 12 GHz |
| Signal bandwidth | 100 MHz |
| Reference input power | -20 dBm |
| 5G NR waveform | NR-FR1-TM3.1, 100 MHz, 30 kHz SCS, FDD |
| Baseline antenna array | 4 dipoles |
| Target objective | Increase EIRP above 42 dBm while keeping EVM around or below 3% |

# Reference Chain Components

| # | Component | Key characteristics | Role in chain |
| --- | --- | --- | --- |
| 1 | IF filter (BFIC #1) | - Inverse Chebyshev bandpass; - order 5; - passband 3.95 to 4.05 GHz; - passband attenuation 3 dB | Removes DAC/image/spur content around the 4 GHz IF before RF conversion. |
| 2 | Driver amplifier (BFIC #2) | - Touchstone file CMD240withNF.s2p; - OIP3 28 dBm; - OP1dB 19 dBm; - OPsat 22 dBm | Amplify the input signal, with low noise contribution. |
| 3 | RF tuner / upconverter (BFIC #3) | - LO 8 GHz; - output RF 12 GHz; - Gain 6 dB; - NF 3 dB; - OIP3 32 dBm | Upconverts the 4 GHz IF waveform to the 12 GHz RF carrier. |
| 4 | Wilkinson corporate splitter (BFIC #4) | - S-parameters data WilkinsonSplitterData - 4-way split | Distributes the RF signal to the PA/antenna paths with realistic insertion loss and S-parameters. |
| 5 | Power amplifier | - Gain 25 dB; - NF 6 dB; - OIP3 45 dBm; - OP1dB 33 dBm; - OPsat 37.5 dBm | Dominant EIRP driver and nonlinearity source; soft GaN compression. |
| 6 | Antenna array | - Dipole designed at 12 GHz; - Baseline N=4 dipoles; | Terminates the RF budget with radiating element gain/impedance for the representative path. |